BMS Document Information:

| BMS Document Number: | 4T01 |
| --- | --- |
| BMS Document Title: | Marketing Requirements Document Template |
| BMS Document Type: | Template |
| BMS Usage Requirement: | Optional |
| BMS Document Owner: | Dietmar Tandler |
| BMS Document Completed by: | Klaus Coenning |
| BMS Document Revision: | 1.0 Revision number is reverted back to Version 1.0 to align with the new Product Development Policy BMS SharePoint site created as part of the Simplification Initiative. |
| Date Document Completed: | 27-Aug-2014 |

Instructions for Using this Record Template

This template, when completed, will become a Record as outlined in Sections 4.2.3 and 4.2.4 of the TL9000 Quality Management System Requirements Handbook, Release 5.0.

When using this template

Do not change the BMS Template Information (this page).

Update the template on starting on Page 1.

Control of records is outlined in Sections 4.2.3 and 4.2.4 of The TL9000 Quality Management System Requirements Handbook, Release 5.0.

Populate Detailed Record Revision History and  Footer information to reflect record revision and date.

Released versions of the record should begin with Version 1.0.

Use BMS Styles for formatting headers, body text, bullets and numbering.

Rename document (e.g. RCATS Release 9.8 MRD, Revision 1.0).

Delete Red text as it is instructions on how to complete the record.

Marketing Requirements Document

for NSA, RtSA, and NR TM

Record Revision History

| Rev | Date | Editor(s) | Description of change |
| --- | --- | --- | --- |
| 3.0 | 15-Oct-19 | Jin-Ook Kim | Adding the TM requirements |

Table of Contents

Contents

1	Process map	3

2	Purpose	3

3	Scope	3

4	Audience	3

5	Definitions	4

6	References	4

7	Roles and Responsibilities	4

8	Value Proposition	4

9	Description of Users	4

10	Use Case	5

10.1	Interfernece Hunting over CPRI interface	5

11	Requirements	5

11.1	RtSA POI	5

11.2	NSA mode	5

12	Market Windows	10

12.1	NSA (5G and LTE)	10

12.2	RtSA POI	10

# Process map

Project Plan

Design & Implementation

PDP

Tech. Doc.

System Architecture

Verification & Validation

System
Integration

Software

Hardware

FPGA

Mechanical

Requirements

PLM

Operations

Service

Marketing

# Purpose

Purpose of this document is to specify the requirements for 5G NR NSA test capability and enhancement target of POI of RtSA.

# Scope

This project is applied to CA5000-F002 model in terms of hardware modification but can be applied to all CA5G product family in terms of software feature.

# Audience

This document is targeted for the following functions/teams:

- R&D

- NPI

- Operation

- Service & TAC

- Tech Pub

# Definitions

| Term | Definition |
| --- | --- |

# References

# Roles and Responsibilities

- R&D: Yoo-Chul Shin, Wei Chen, Sang-Gu Kang , Sang-Young Jung, Jae-Yong Park, Hyuck-In Kim, Nick G.

- QA: YongSung Jun

- PLM: JaeGab Lee, Eduardo, Reza Vaez Ghaemi

- GTM: Kashif Hussein, Jennifer Faircloth

- PMO: Jace Kim

- Tech Pub: Jay Kim

- Service: Su-Hee Park

- Ops: Pyeong-Soo Kim, Young-In Kim

- Sales: Joe Chou, Teressa Watts

# Value Proposition

- 5G NR launch in being made on NSA mode. Therefore, the market demand for a OTA solution that can cover both LTE and NR looks substantial.

- Especially for 5G NR operators on FR2 band, simple and cost effective antenna solution that can cover both FR1 and FR2 is highly demanded.

# Description of Users

Installation, Maintenance, and optimization engineers responsible for 5G NR NSA mode operation.

# Use Case

# Requirements

## RtSA POI

### Definition

#### Enhancement of RtSA POI

### Requirements

#### POI Target

### License Policy

#### Free enhancement

## NSA mode

### Hardware modification

#### Direction of CA5G RF frontend modification is to add a supplemetary N-type port to allow CA5G can receive RF signal under 6 GHz through a separate antenna while the main k-type port is receiving mm Wave signal on FR2 band through a mm Wave antenna.

#### Two possible examples. R&D to choose the best direction to get reliability of new hardware and also can secure the effeciency of hardware upgrade from CA5000-f002.

### 5G NR - NSA OTA Analyzer overview

#### Key functions are the same with current NR OTA

##### Carrier Scanner

##### Signal Analyzer

##### Route Map

#### NSA OTA supports two operation mode, e.g. Fast and Normal.

#### NSA OTA supports extended mode that shows multiple PCI with associated signal information.

#### Functional Map

| Operation Mode | Carrier Scanner | Signal Analyzer | Routemap |
| --- | --- | --- | --- |
| Fast | Primary PCI and RSRP of each carrier | Primary PCI, RSRP, and Beam index (NR) | Route map with fast mode Carrier Scanner or Signal Analyzer |
| Normal | ++ Channel Power, Time Error, Freq Error, RS EVM (for LTE), and PBCH EVM (for NR) | ++ NR: SINR and RSRQ, multi Beam index of multi PCI LTE: ID Scanner or OTA Control | Route map with normal mode carrier scanner or Signal analyzer |
| Pop-up | PCI Scanner | PCI Scanner | Not supported |

#### Number of carriers: User can confiure up to 8 carriers

##### Typical use case for NSA operation is to configure 2x LTE are for the control plane and 1x NR carrier for data plane.

##### In the future, NR on FR1 band can be used for control plane in addition to or on behalf of the LTE control plane.

##### Carrier Aggregation on NR will involves more NR data carriers. Therefore, the number of configurable NR carrier shall be expanded.

### Carrier Scanner

#### Definition

##### Carrier Scanner provides information that helps to identify the coverage of each BS by showing dominant PCI and RSRP.

#### Operation

| Operation Mode | Carrier Scanner |
| --- | --- |
| Fast | Primary PCI and RSRP of each carrier |
| Normal | ++ Channel Power, Time Error, Freq Error, RS EVM (for LTE), and PBCH EVM (for NR) |

##### Fast Mode

###### Carrier scanner shows the strongest PCI of each carrier regardless of technology.

###### Scan Time per carrier should not longer than 300 ms.

##### Normal Mode

###### It provides additional parameters shown on the above table

###### Enable Magnifier for each carriers

###### Scan Time per carrier should not longer than 600 ms.

##### Pop-up

###### Magnifier pops up beam analyzer for NR and ID Scanner for LTE

###### User can pop-up an additional window showing multi PCI staus of each carrier, or all at once (if applicable)

##### Support Data logging

### SIgnal Analyzer

#### Definition

##### Signal Analyzer provides information that helps to identify the beam coverage and signal quality with reliability.

#### Operation

| Operation Mode | Signal Analyzer |
| --- | --- |
| Fast | Primary PCI, RSRP, and Beam index (NR) |
| Normal | ++ NR: SINR and RSRQ, multi Beam index of multi PCI LTE: ID Scanner or OTA Control |

##### Fast Mode

###### Primary PCI and RSRP for LTE Carrier

###### Primary PCI, RSRP, and beam index for NR Carrier

###### Scan Time per carrier should not longer than 600 ms.

##### Normal Mode

###### For 5G NR Signal, it shows Beam Analyzer screen that shows 8 strongest beams from any of available PCI (multi PIC)

###### For LTE, it shows ID Scanner screen or Control Channel selectively

##### Pop-up

###### User can pop-up a window showing multi PCI staus of each carrier or all at once.

#### GUI

##### Fast mode shows whole information on a single chart.

##### Normal mode consists of multiple charts. Please refer to the preferable combination of charts.

###### 2x LTE + 1x NR (FR2) scenario

###### 1x LTE + 1x NR (FR2) scenario

###### 2x LTE + 2x NR (FR2) scenario

###### 1x NR (FR1) + 1x NR (FR2) scenario

##### User can configure the number of chart to show on the screens

##### Should support Data logging

### NSA OTA Route Map

#### Definition

##### Route Map works with both Carrier Scanner and Signal Analyzer.

##### Support Fast and Normal mode

##### Multi PCI is supported only when it is running Signal Analyzer with Normal mode

#### Operation

| Operation Mode | Routemap |
| --- | --- |
| Fast | Route map with fast mode Carrier Scanner or Signal Analyzer |
| Normal | Route map with normal mode carrier scanner or SIgnal analyzer |

##### User can choose one of the above mode to diplay the result on the map.

## Test Mode

### What is the TMs

### 3GPP identifies test signals for test cases.  3GPP defined the test models for 5G NR gNB transmitter testings in TS38.141 for confirmance testings.

### General requirements

#### The reference 3GPP documentation should be 2019-09 for TS38.104 and 141

#### FDD and TDD should be supported together

#### Following test cases should be supported in FR1 and FR2 bands

##### BS output power

##### Unwanted emissions

###### Occupied BW

###### ACLR

###### Operating band unwanted Emissions

###### Spurious Emissions

##### Signal Quality

###### Transmitt ON/Off power

###### Time alignment Error

###### Frequency Error

###### EVM for QPSK, 16QAM, 64QAM and 256QAM

#### BS output power, Occupied BW, ACLR, operating band unwanted emissions and spurious mission should be have spectrum waveform

#### Transmitt ON/Off power should have PvsT trace

#### EVM should be have the constellation diagram

#### TM analyzer should have an auto trigger spectrum analyzer for TDD

#### The results of each test cases should be met the 3GPP recommendation spec boundary.

#### Following test models and test cases should be supported in FR1 and FR2 bands

### License:  this is new SW options feature (option #TBD)

# Market Windows

## NSA (5G and LTE)

- Alpha availability: 16-Dec-2019

- Beta availability: 09-Mar-2020

- FCS availability: 27-Mar-2020

## RtSA POI

- FCS availability: 27-Mar-2020

## TM

- Pre-Alpah availability: 16-Dec-2019

- FCS availability: 27-Mar-2020